<INSERT ORGANISATION NAME> BOILER SAFETY CHECKLIST

| BOILER SAFETY CHECKLIST |  |  |  |
| --- | --- | --- | --- |
| Maintenance |  | Yes | No |
| Are all boilers subject to a systematic maintenance regime? (Normally annual, these checks will verify that appliances are on the right setting, burning correctly at the correct pressure, harmful gases are being removed safely to the air outside, ventilation is sufficient and all the safety devices are working.) |  |  |  |
| If you have multiple boilers, have you listed them to ensure all are properly maintained? |  |  |  |
| CO Hazard |  |  |  |
| Incomplete combustion can lead to lethal carbon monoxide (CO) formation and build up: |  |  |  |
| Does the boiler have sufficient ventilation to operate safely? |  |  |  |
| Are ventilation openings clear and unobstructed? |  |  |  |
| Is the flue in good condition so that exhaust gases pass safely to the outside? |  |  |  |
| Have you fitted carbon monoxide detectors in boiler rooms and any accommodation in which people sleep? |  |  |  |
| Gas Boilers |  |  |  |
| Are all contractors who work on gas boilers Gas Safe registered? |  |  |  |
| Do you hold a current gas safety certificate for any boilers and other equipment that’s in residential accommodation? |  |  |  |
| Do your staff know the 0800 111 999 emergency number for reporting any suspected gas leak? |  |  |  |
| Do staff know the other do’s and don’ts for dealing with a gas leak (see below)? |  |  |  |
| Do Don’t |  |  |  |
| - Put out any naked flames. - Open all the doors and windows. - Turn off the gas supply at the meter. - Leave it switched off until you’re sure it’s safe to turn on. | ✘ Smoke ✘ Light matches or cigarette lighters. ✘ Turn any electrical equipment on or off. ✘ Try to put out a gas fire out with a fire hose it back on again. or portable extinguisher. |  |  |
| Asbestos |  |  |  |
| Do you know whether your boiler/pipe work lagging contains asbestos? (If it does, you will be a duty holder under the Control of Asbestos Regulations 2012: you’ll need to have a written management plan and make sure the asbestos remains in good condition and is not disturbed by other work.) |  |  |  |
| Steam Boilers |  |  |  |
| Is your boiler regularly examined under a written scheme as required by the Pressure Systems Safety Regulations 2000? |  |  |  |
| Is it fitted with standard safety devices such as steam pressure gauge and low water alarms? |  |  |  |
| Is the maximum permissible working pressure clearly marked? |  |  |  |
| Are water gauge glasses shielded to prevent anyone nearby being injured in the event of failure? |  |  |  |
| Do you have written instructions for safe boiler start up, blow down and shut down? |  |  |  |
| Records |  |  |  |
| Do you record all boiler checks and maintenance? |  |  |  |
| Do you act promptly to rectify any faults found? |  |  |  |