BOILING LIQUID EXPANDING VAPOUR EXPLOSION (BLEVE) FACTSHEET

Gases expand within the cylinder as heat is applied

Stage One

A fire in the vicinity of the cylinder acts on the

liquid, causing it to boil. As the temperature

increases the contents turn to vapour

and expand, increasing the pressure within

the cylinder.

Jet flame burns off

escaping vapour

Gases cause pressure release valve to operate

# Stage Two

The pressure increase causes the pressure release valve to operate. Escaping vapours are ignited by the heat source and a jet flame develops.

Gases occupy a greater volume of the

cylinder and continue to expand.

As more of the fuel is burned, the level of the liquid within the cylinder begins to fall.

# Stage Three

The level of the liquid in the cylinder continues to reduce as more fuel is burned. The fire begins to act directly on the gases within the cylinder increasing the rate of expansion. At the same time the materials of the cylinder are heated and the yield strength reduces in the heat.

# Stage Four

The cylinder ruptures and fails catastrophically. The remaining liquid contained within the cylinder immediately expands to become a vapour. All remaining fuels are ignited and the resulting explosion produces a large fire ball and pressure wave. The consequences are similar in effect to a bomb. Debris will be ejected and the pressure wave is likely to destroy structures in the vicinity.