<INSERT ORGANISATION NAME> COMPRESSED AIR CHECKLIST

| COMPRESSED AIR CHECKLIST |  |  |
| --- | --- | --- |
| Compressed Air Hazards | Yes | No |
| Have you recognised compressed air hazards and included them in your risk assessments? Check for the 7 key dangers highlighted by the HSE: - It can enter body orifices such as the mouth, ears and anus, causing severe and often fatal injuries. - At high pressure it can penetrate the skin. - Particles or oil carried in an air jet can damage the eyes. - Oil-coke deposits in a system can spontaneously ignite and cause an explosion. - Tanks and other things that hold compressed air (e.g. tyres) can explode violently if they are damaged or fail under pressure. - Dirty or ‘wet’ air can lead to corrosion and blocked valves that can make pneumatic systems unsafe. - Hoses that fail under pressure can snake/whiplash, causing injury. |  |  |
| Basics − All Compressors |  |  |
| - Have you appointed a responsible person to make sure compressors are operated and serviced in accordance with the manufacturer’s guidance? |  |  |
| - Are they fitted with a pressure gauge and a safety valve? |  |  |
| - Is the drive between the motor and the compressor guarded so that no one can touch moving parts? |  |  |
| - Are they sited away from people to minimise the noise hazard, for example, in a separate plant room? |  |  |
| - Is there clear access for maintenance checks? |  |  |
| - Are they regularly checked by a competent person (the interval will be set in your written scheme of examination under the Pressure Systems Safety Regulations 2000)? |  |  |
| - Do you check hoses and pipe work as well as the compressor itself? |  |  |
| - Do you have records of all competent person checks and the action you took to rectify any faults found? |  |  |
| Using Compressed Air for Tyre Inflation |  |  |
| - Do you use cages when inflating any commercial vehicle tyre above 15psi? |  |  |
| - Can your staff recognise divided and split rim wheels – and the associated hazard of the wheel parts separating violently during tyre inflation? |  |  |
| - Do the air line controls allow the user to stand well back from the tyre being inflated? |  |  |
| - Where there is an explosion risk, have you marked the danger area with floor markings? |  |  |
| Training |  |  |
| - Have you trained all users to appreciate the hazards and given them practical advice on how to work safely with compressed air, for example, ‘never lift a pneumatic tool by the hose’? |  |  |
| - Have you prohibited the use of compressed air for the cleaning of clothes and for leak testing? |  |  |
| Technical |  |  |
| - Have you fitted silencers on pneumatic equipment exhausts? |  |  |
| - Where you use air hoses that are >10mm bore, >10m long or at >7bar pressure, do you use self-venting connectors so that downstream pressure is released before they can be disconnected? |  |  |
| - If you use compressed air for part or equipment cleaning, have you considered safer alternatives (e.g. vacuuming) and do you use a ‘safety’ type nozzle? |  |  |
| - If you have high- and low-pressure systems, are the connectors incompatible or at least colour-coded to minimise the risk of low-pressure equipment being connected to high pressure air? |  |  |